SAMPLE TENDER SPECIFICATION TEXTS

Wall construction works with TRIQBRIQ wall system WS25

Preliminary remarks

The German VOB Part C, ATV DIN 18334 “Carpentry and timber construction works” applies. Planning and execution are carried out in accordance with the general building authority approval (abZ) No. Z-9.1-912 dated 8 April 2024. The TRIQBRIQ construction site manual is the basis of execution (current version: triqbriq.de/downloads).

The solid timber walls shall be erected using the TRIQBRIQ wall system WS25 by TRIQBRIQ, on which the execution planning and this tender are based.

The system is based on the solid timber blocks WS25B25 and WS25B50, the flexural beams (lintels) WS25B200B and WS25B400B and the columns WS25B200S and WS25B400S. The blocks are laid in stretcher bond and connected glue-free exclusively by tenons and beech wood dowels – fully demountable and reusable without residues. Waste-free delivery on pallets with returnable lashing straps.

Dowels are included in the scope of delivery; the locking tool is available for rent for the duration of the construction project (driven by a compressor to be provided on site). Moisture protection during construction is the contractor’s responsibility (DIN 68800). Details on block storage and rental of the locking tools are set out in the construction site manual.

'…' = to be completed by the planner on a project-specific basis. (Contingency item) = execution only upon separate instruction by the client.

Building physics data of the system are available in the download portal: https://triqbriq.de/downloads

1. Preparation and anchoring

Item 1.10 Levelling/bedding course, grout concrete, for TRIQBRIQ WS25 wall, t=25 cm

Provide a levelling course of grout concrete on the substrate as an even, level bearing surface for the solid timber wall.

Purpose:	height adjustment; level surface in longitudinal and transverse direction; levelling of substrate irregularities.

Width:	25 cm

Thickness:	min. 1 cm, max. 3 cm

Quantity: '…' m	UP: ......................	TP: ......................

Item 1.20 Horizontal damp-proof course, for TRIQBRIQ WS25 wall, w=25 cm (contingency item)

(Note: The damp-proof course shall be designed and specified on a project-specific basis.)

Install a horizontal damp-proof course as waterproofing against capillary rising damp in accordance with DIN 18533.

Purpose:	protection of the solid timber wall against rising damp

Material:	'…' (acc. to DIN 18533, to be specified by the specialist designer)

Width:	25 cm

Quantity: '…' m	UP: ......................	TP: ......................

Item 1.30 Acoustic decoupling strip, for TRIQBRIQ WS25 wall, w=25 cm (contingency item)

(Note: Design and material selection by the responsible specialist designer; to be planned and specified on a project-specific basis.)

Lay a load-bearing, settlement-resistant insulation strip beneath the vertical, load-bearing timbers of the TRIQBRIQ wall system.

Purpose:	acoustic decoupling wall/floor slab

Material/properties:	as specified by the specialist designer

Width:	25 cm

Location:	'…'

Quantity: '…' m	UP: ......................	TP: ......................

2. Walls TRIQBRIQ wall system WS25

Item 2.10 External wall, TRIQBRIQ WS25, t=25 cm

Construct external wall of solid timber blocks of the TRIQBRIQ wall system WS25 in stretcher bond, glue-free connection by tenons and beech wood dowels, in accordance with general building authority approval Z-9.1-912.

Scope of work:

- laying of the blocks in stretcher bond, top of wall level

- top course without tenons (blocks orderable tenon-free; level wall head as bearing for the floor slab)

- dowelling (2 beech wood dowels per block), flush with the wall surface

- corner formation in bond

- forming of openings up to 2.5 m² individual size (measured over)

- connection to existing components incl. movement joint

- cutting and fitting pieces; length/height adjustment with timber planks

- temporary bracing by raking shores incl. provision

- connectors and fixings as per structural requirements (e.g. steel angles, tie anchors)

Purpose:	load-bearing external wall

System:	TRIQBRIQ wall system WS25

Blocks:	WS25B50 (full), WS25B25 (half)

Material requirement:	8 pcs WS25B50 per m² wall area; plus 2 dowels per block

Material:	softwood, glue-free

Strength class:	min. C24

Wall thickness:	25 cm

Measurement:	m² wall area; openings up to 2.5 m² individual size are measured over (ATV DIN 18334, section 5)

Quantity: '…' m²	UP: ......................	TP: ......................

Item 2.20 Internal wall, TRIQBRIQ WS25, t=25 cm

Construct internal wall of solid timber blocks of the TRIQBRIQ wall system WS25 in stretcher bond, glue-free connection by tenons and beech wood dowels, in accordance with general building authority approval Z-9.1-912.

Scope of work:

- laying of the blocks in stretcher bond, top of wall level

- top course without tenons (blocks orderable tenon-free; level wall head as bearing for the floor slab)

- dowelling (2 beech wood dowels per block), flush with the wall surface

- corner formation in bond

- forming of openings up to 2.5 m² individual size (measured over)

- connection to existing components incl. movement joint

- cutting and fitting pieces; length/height adjustment with timber planks

- temporary bracing by raking shores incl. provision

- connectors and fixings as per structural requirements (e.g. steel angles, tie anchors)

Purpose:	internal wall

Load-bearing function:	'…' (load-bearing / non-load-bearing)

System:	TRIQBRIQ wall system WS25

Blocks:	WS25B50 (full), WS25B25 (half)

Material requirement:	8 pcs WS25B50 per m² wall area; plus 2 dowels per block

Material:	softwood, glue-free

Strength class:	min. C24

Wall thickness:	25 cm

Measurement:	m² wall area; openings up to 2.5 m² individual size are measured over (ATV DIN 18334, section 5)

Quantity: '…' m²	UP: ......................	TP: ......................

Item 2.30 Gable slopes, TRIQBRIQ WS25, t=25 cm

Form gable slopes on external and internal walls in the attic storey by angled cutting of the solid timber blocks.

Purpose:	adaptation of the wall to the roof pitch

Execution:	angled cutting on site, roof pitch as per design

Wall thickness:	25 cm

Location:	attic storey

Quantity: '…' m	UP: ......................	TP: ......................

Item 2.40 Lintel, TRIQBRIQ flexural beam WS25B200B, l=200 cm, t=25 cm

Span the opening in walls of solid timber blocks with flexural beam WS25B200B in accordance with general building authority approval/type structural analysis.

Purpose:	load-bearing spanning of the wall opening

Type:	WS25B200B, l=200 cm

Lintel height:	25 cm

Wall thickness:	25 cm

Max. structural opening width:	150 cm

Bearing length:	min. 25 cm each side

Connection:	tenons/blind holes and beech wood dowels, as wall system

Pieces per opening:	1

Quantity: '…' pcs	UP: ......................	TP: ......................

Item 2.50 Lintel, TRIQBRIQ flexural beam WS25B400B, l=400 cm, t=25 cm

Span the opening in walls of solid timber blocks with flexural beam WS25B400B in accordance with general building authority approval/type structural analysis.

Purpose:	load-bearing spanning of the wall opening

Type:	WS25B400B, l=400 cm

Lintel height:	25 cm

Wall thickness:	25 cm

Max. structural opening width:	350 cm

Bearing length:	min. 25 cm each side

Connection:	tenons/blind holes and beech wood dowels, as wall system

Pieces per opening:	1

Quantity: '…' pcs	UP: ......................	TP: ......................

Item 2.60 Column, TRIQBRIQ WS25B200S, l=200 cm, 25 × 25 cm

Install column WS25B200S as a load-bearing element in walls and structures of solid timber blocks, in accordance with general building authority approval/type structural analysis.

Purpose:	load-bearing column

Type:	WS25B200S, l=200 cm

Cross-section:	25 × 25 cm

Material:	softwood, grading class S10 (DIN 4074-1)

Strength class:	C16 or C24 (DIN EN 338), as per structural requirements

Special lengths:	available on request (custom production)

Installation:	lifting aid (e.g. mini crane) recommended

Quantity: '…' pcs	UP: ......................	TP: ......................

Item 2.70 Column, TRIQBRIQ WS25B400S, l=400 cm, 25 × 25 cm

Install column WS25B400S as a load-bearing element in walls and structures of solid timber blocks, in accordance with general building authority approval/type structural analysis.

Purpose:	load-bearing column

Type:	WS25B400S, l=400 cm

Cross-section:	25 × 25 cm

Material:	softwood, grading class S10 (DIN 4074-1)

Strength class:	C16 or C24 (DIN EN 338), as per structural requirements

Special lengths:	available on request (custom production)

Installation:	lifting aid (e.g. mini crane) recommended

Quantity: '…' pcs	UP: ......................	TP: ......................